# Tests

| Test # | Test procedure / description | Expected Outcome | Actual Outcome & Remarks | Pass / Fail |
| --- | --- | --- | --- | --- |
| 1 | Smoke test the firewall: - apply the firewall: - sudo ./firewall.sh - run the test script: - sudo ./test.sh [peer ip address] - run the test script from the peer computer: - sudo ./test.sh [firewall ip address] - list firewall rules: - sudo iptables -L -v -n | - During step 1, nothing is printed. - During step 2, all tests except for those under the "### testing firewall rules of $address ###" heading should pass. - During step 3, all tests under the "### testing firewall rules of $address ###" heading should pass - During step 4, all firewall rules are listed. most rules have a non-zero packet count. all user chains have a non-zero packet count. | As expected | Pass |
| 2 | Test DNS, HTTP and HTTPS: - apply the firewall: - sudo ./ firewall.sh - open a web browser and go to a website that uses the HTTP protocol, then go to another site that uses a HTTPS protocol. - list firewall rules: | When listing firewall rules: - all DNS rules associated with UDP protocols should have non-zero packet counts. - WWW_CLIENT rules associated with the WAN interface should have non-zero packet counts. - the WWW_CLNT and DNS chains should have non-zero packet counts. | As expected | Pass |
| 3 | Test SSH to peer from firewall: - apply the firewall: - sudo ./ firewall.sh - set up an SSH server on a peer. - SSH to the peer from the firewall. - list firewall rules. | When SSH-ing to the peer from the firewall: - A connection should be established; the program should not hang. When listing firewall rules: - SSH_CLNT rules associated with the WAN interface should have non-zero packet counts. - SSH_CLNT chain should have a non-zero packet count. | As expected | Pass |
| 4 | Test HTTP into the firewall from peer: - apply the firewall: - sudo ./ firewall.sh - set up a web server on the firewall. - connect to the web server from peer. - list firewall rules. | When connecting to the firewall from a peer with a web browser: - A connection should be established; the web browser should not hang. it should display a web page. When listing firewall rules: - SSH_SVR rules associated with the WAN interface should have non-zero packet counts. - SSH_ SVR chain should have a non-zero packet count. | As expected | Pass |
| 5 | Test SSH into the firewall from peer: |  |  |  |
| 6 | Test HTTP into firewall from firewall: |  |  |  |
| 7 | Test SSH into firewall from firewall: |  |  |  |

## Screenshots

Figure 1 test 1, all user chains have non-zero values, and most firewall rules also have non-zero values

Figure 2 test 2, all DNS rules have non-zero packet count, and www_clnt rules associated with wlan0 interface has a non-zero packet count

Figure 3 test 3, ssh connection established successfully

Figure 4 test 3, all  SSH_CLNT rules associated with wlan0 interface as well as the SSH_CLNT chain have non-zero packet counts

Figure 5 test 4, browser can connect to the firewall, and it is displaying a web page delivered by the web server on the firewall

Figure 6 test 4, WWW_SVR rules associated with wlan0 as well as the WWW_SVR chain display non-zero packet counts